Final Term

12. Number system-> Binary to Decimal and Decimal to Binary

Binary -> 2 base ( 11012 )                         Decimal -> 10 base ( 1310 )

Hexadecimal -> 16 base ( 62C )               Octal -> 8 base ( 1548 )

Binary to Decimal convert:

Find the decimal value of 1110012 :

Decimal to Binary convert:

Find the binary value of 1310  :

*** Down -> top (নিচ থেকে উপরের দিকে সংখ্যা সাজানো)

Floating Number Decimal to Binary convert:

Top -> Down

13. Benefits of High level Language (HLL)

- Closer to Natural Languages

  - C++

  - FORTRAN scientific computation

  - COBOL	business data processing

- Increased Programmer productivity

  - Small and concise code

  - Easy and Quick Programming

- Independent of Hardware Platform

  - Compiler converts HLL code to target code

14. Define Instruction Set Architecture (ISA) with example

- Different CPUs implement different sets of instructions.  The set of instructions a particular CPU implements is an Instruction Set Architecture (ISA).

  - Examples: Intel 80x86 (Pentium 4), IBM/Motorola PowerPC (Macintosh), MIPS, Intel IA64, ...

15. Explain Reduced Instruction Set Computing.

RISC (Reduced Instruction Set Computer) is a CPU design that uses a small number of simple and fast instructions to improve performance and efficiency.

RISC philosophy (John Cocke IBM, David A. Patterson, John L. Hennessy, 1980s) – Reduced Instruction Set Computing

  - Keep the instruction set small and simple, makes it easier to build fast hardware.

Features of RISC:

  - fixed instruction lengths

  - load-store instruction sets

  - limited addressing modes

  - limited operations

16. Define Microprocessor without Interlocked Pipeline Stages (MIPS) Architecture. Why MIPS instead of Intel 80x86?

MIPS – semiconductor company that built one of the first commercial RISC architectures.

Instead of Intel 80x86:

  - MIPS is simple, elegant.  Don’t want to get bogged down in gritty details.

  - MIPS widely used in embedded apps, x86 little used in embedded, and more embedded computers than PCs

  - NEC, Silicon Graphics, Nintendo, Sony etc.

17.  Assembly Variables : Registers

a. Question: Drawback: Since registers are in hardware, there are a predetermined number of them

Solution: MIPS code must be very carefully put together to efficiently use registers.

Why Is This a Drawback?

1. Limited Number of Registers

2. Performance May Decrease: More memory access → More load/store instructions → Slower execution

3. Less Flexibility

b. Question:  32 registers in MIPS. Why 32?

Smaller is faster

In MIPS architecture, there are 32 general-purpose registers. This design choice was made for a balance between performance, hardware cost, and simplicity.

Main reasons for choosing 32 registers:

1. Efficient Instruction Encoding

2. Good Balance Between Cost and Performance

3. Supports Compiler Optimization

18. MIPS Assembly Language

Arithmetic -> all                               Data transfer -> load, store

Logical -> and, or                             Conditional branch -> all

Unconditional jump -> jump, jump register

19. Format (R,I,J)

20. MIPS register

21. Assembly Variable : Registers

- Registers are numbered from 0 to 31

- Each register can be referred to by number or name

- Number references:

$0, $1, $2, … $30, $31

*** By convention, each register also has a name to make it easier to code

- For now:

$16 - $23	    	$s0 - $s7

(correspond to C variables)

$8 - $15	   	 $t0 - $t7

22. C , Java variables vs registers

| Variables (C/Java) | Registers |
| --- | --- |
| 1. Declared by programmer | 1. Built into CPU hardware |
| 2. Stored mainly in RAM | 2. Stored inside CPU |
| 3. Slower access | 3. Very fast access |
| 4. Can create many variables | 4. Limited number (e.g., 32 in MIPS) |
| 5. Example: int fahr, celsius; Char a,b,c,d,e; | 5. Example: register -> $ (dollar sign) |

23. Assembly Instruction Short note:

- In assembly language, each statement (called an Instruction), executes exactly one of a short list of simple commands

- Unlike in C (and most other High Level Languages), each line of assembly code contains at most 1 instruction

- Instructions are related to operations (=, +, -, *, /) in C or Java

24. Addition and Subtraction of Integers:

- Addition in Assembly

  - Example:  add	$s0, $s1,$s2 (in MIPS)

Equivalent to:	   a = b + c (in C)

where MIPS registers $s0, $s1, $s2 are associated with C variables a, b, c

- Subtraction in Assembly

  - Example:	sub	$s3, $s4,$s5 (in MIPS)

Equivalent to:	d = e - f (in C)

where MIPS registers $s3, $s4, $s5 are associated with C variables d, e, f

25. Add Instruction:

- Instructions, like registers and words of data, are 32 bits long

- Arithmetic Instruction Format (R format):

26. R-format example -> Machine code convert.

i. Assembly to Binary                       ii. Binary to Assembly

Question:  add $t0,   $s1,    $s2

27. Addition and Subtraction of Integers

- How do the following C statement?

a = b + c + d - e;

- Break into multiple instructions

add $t0, $s1, $s2                                             # temp = b + c

add $t0, $t0, $s3                                              # temp = temp + d

sub $s0, $t0, $s4                                               # a = temp - e

- Notice: A single line of C may break up into several lines of MIPS.

- Notice: Everything after the hash mark on each line is ignored (comments)

28. Addition and Subtraction of Integers

- How do we do this?

f = (g + h) - (i + j);

- Use intermediate temporary register

add $t0,$s1,$s2		# temp = g + h

add $t1,$s3,$s4		# temp = i + j

sub $s0,$t0,$t1		# f=(g+h)-(i+j)

29. Register Zero

- One particular immediate, the number zero (0), appears very often in code.

- So we define register zero ($0 or $zero) to always have the value 0; eg

add $s0, $s1, $zero     (in MIPS)

f = g (in C)

where MIPS registers $s0, $s1 are associated with C variables f, g

- defined in hardware, so an instruction

add   $zero, $zero, $s0

30. Immediates

- Immediates are numerical constants.

- Add Immediate:

add $s0, $s1, 10   (in MIPS)

f = g + 10 (in C)

where MIPS registers $s0, $s1 are associated with C variables f, g

*** 27, 28, 29, 30 -> simple code segment দেওয়া থাকবে  Assembly language convert করতে হবে .